# Assignment 1

## Task A — Voltage Divider Analysis and Measurement Module

- Purpose of voltage divider: A voltage divider scales down an input voltage so it lies within the ADC input range (0–5 V) of the Arduino. It is used in the digital multimeter.

- Circuit:
 Components-:     Arduino UNO, Breadboard, Two resistors, Jumper wires.

(R1=R2=10 k ohm)

- Voltage Divider Formula:
Vout=Vin(R2/R1+R2)

- ADC Reading to Voltage Conversion:
We provided Vin(Vout at R2) to A0. Arduino internal ADC converts it into a number (ranging 0 to 1023). Then our Vmeasured(Vout at R2) we calculate it by this formula: Vmeasured=adc*5.0/1023.

- Code with explanation:

- Three cases:

Case1: (R1=R2=10kohm)
Vout=5*(10/10+10)=2.5V
ADC  expected value=1023*(2.5/5)=512

Case2: (R1=4.7kohm, R2=10kohm)
Vout=5*(10/4.7+10)=3.4V
ADC  expected value=1023*(3.4/5)=696

Case3: (R1=1kohm, R2=15kohm)
Vout=5*(15/1+15)=4.69V
ADC  expected value=1023*(4.69/5)=959

- Serial monitor Output:
Case1:  
Case2: 
Case3:

- Comparison: 
Case1: Theoretical Vout (V):2.50
             Tinkercad Measured (V):2.498
Case2: Theoretical Vout (V):3.40
             Tinkercad Measured (V):3.402
Case3: Theoretical Vout (V):4.69
             Tinkercad Measured (V):4.687

- Observations:
Small errors due to ADC quantization.
There is noise in ADC readings.
We have Limited resolution (4.88 mV per step).

## Task B — Capacitance Measurement Using RC Time Constant

1. Principle: A capacitor charged through a resistor follows: Vc(t)=Vs(1-e^-(t/RC)).
                             At t=RC  Vc=0.632Vs  that is 63%. 
                            RC is time constant. At 63% of supply voltage C=t/R.

2.Circuit: Components used:
                    Arduino UNO
                    Resistor R (here 10 kΩ)
                    Capacitor C (test values)
                    Breadboard
                    Jumper wires

3. Why 63% is used : at t=RC  Vc=0.632Vs i.e.63% and it is independent of R and C values.Also its a standard method used in instruments.

4. ADC calculation for 63% : ADC63%​=0.632×1023=647

5.Code:

6.Test Cases:

Case1: C=10uF 
t=RC=10k×10u=0.1s=100,000us
measured:
Case2: C=100uF 
t=RC=10k×100u=1s=1,000,000us
measured:

7.Observation: 
Case1: C=10uF
Expected time=100,000us
Measured time=100516us

Case2: C=100uF
Expected time=1,000,000us
Measured time=1004560us

7.Error Sources:
Capacitor tolerance (+-10 to 20%)
ADC quantization error
Threshold approximation (63% that is nearly 647 ADC)
Leakage currents

## Task C — Beginner Ohmmeter Prototype

## 1.Principle:

An ohmmeter does NOT measure resistance directly.
It measures voltage, then calculates resistance using a known resistor.
Same idea as Multimeter.

2.Circuit: Components-:     Arduino UNO, Breadboard, Two resistors(one known let 10Kohm and one unknown), Jumper wires.

3.Working Formula: 
Vout=Vin(Runknown/Runknown+Rknown)
Arduino measures Vout
Vin given that is 5V 
Rknown given
Runknown calculated from rearranging eqn.
Runkown=Rknown(Vout/Vin-Vout)

4. ADC Reading to Voltage Conversion:
We provided Vin(Vout at R2) to A0. Arduino internal ADC converts it into a number (ranging 0 to 1023). Then our Vmeasured(Vout at R2) we calculate it by this formula: Vmeasured=adc*5.0/1023.

5.Code:

6.Comparison:

Expected value is what we placed in  tinkercad(here).
Case1: Runknown=2.2kohm(expected value)
Measured Value :
Error:0.314%
Case2: Runknown=4.9kohm(expected value)

Measured Value:
Error:0.187%
Case3: Runknown=7.4kohm(expected value)
Measured Value:
Error:0.027%
 We used 1kohm to 10kohm because if R is too small then there is high current and if R is too large then there will be noise domination by ADC.

7.Measurement uncertainty:
There are various sources of error:
We have Limited resolution (4.88 mV per step).
Tolerance of Resistance.
ADC quantization error.