# Assignment 1 – Tinkercad Support & Learning Resources

# Purpose of This Document

This document is intended to support students who are facing difficulties using Tinkercad Circuits for Assignment 1. It provides step-by-step guidance, curated learning resources, common troubleshooting tips, and best practices to successfully complete the voltage divider, RC circuit, and ohmmeter tasks.

# 1. What is Tinkercad Circuits?

Tinkercad Circuits is an online electronics simulation platform by Autodesk that allows users to build, simulate, and test Arduino-based circuits without physical hardware. It supports real-time simulation of Arduino Uno, basic electronic components, and serial output.

# 2. Getting Started with Tinkercad

Step-by-step setup:
1. Go to https://www.tinkercad.com
2. Sign in using a Google or Autodesk account
3. Select 'Circuits' from the dashboard
4. Click 'Create New Circuit'
5. Drag an Arduino Uno into the workspace
6. Add components from the right-side panel

# 3. Essential Tinkercad Skills for Assignment 1

Students must be comfortable with the following actions:
• Adding and rotating components
• Changing component values (resistors, capacitors)
• Wiring components correctly
• Opening the Code Editor
• Switching from Block Code to Text (Arduino C++)
• Using the Serial Monitor
• Starting and stopping simulation

# 4. Task-wise Resource Mapping

## 4.1 Task A – Voltage Divider

Concept Resources:
• SparkFun – Voltage Dividers Explained
• Khan Academy – Series Circuits

Tinkercad Tutorials:
• Tinkercad Official: Arduino Analog Input
• YouTube: 'Arduino Voltage Divider Tinkercad'

Key Tips:
• Always connect Arduino GND to circuit ground
• Use 5V pin as Vin
• Convert ADC value using: Voltage = (analogRead × 5.0) / 1023

## 4.2 Task B – RC Time Constant (Capacitance)

Concept Resources:
• AllAboutCircuits – RC Time Constant
• MIT OpenCourseWare – Capacitors Basics

Tinkercad Tutorials:
• YouTube: 'RC Circuit Arduino Tinkercad'
• Instructables – Arduino Capacitance Meter

Key Tips:
• Discharge the capacitor before each measurement
• Use millis() or micros() carefully
• Look for ~63% of supply voltage (≈3.15V for 5V)

## 4.3 Task C – Ohmmeter

Concept Resources:
• Electronics-Tutorials – Resistance Measurement
• SparkFun – How to Measure Resistance

Tinkercad Tutorials:
• YouTube: 'Arduino Ohmmeter Tinkercad'

Key Tips:
• Choose a known resistor close to expected unknown value
• Avoid extremely low or high resistance values
• Double-check resistor color codes

# 5. Common Mistakes & Troubleshooting

• Forgetting common ground connection
• Using wrong Arduino pin (digital instead of analog)
• Not switching code editor to Text mode
• Incorrect resistor values
• Serial Monitor not opened after simulation starts
• Using delay() excessively instead of timing functions

# 6. Recommended YouTube Channels

• Paul McWhorter – Arduino Basics
• GreatScott! – Electronics Fundamentals
• Programming Electronics Academy
• EEVBlog – Measurement Techniques

# 7. Submission Expectations Reminder

Students must submit:
• Clear screenshots of Tinkercad circuits
• Arduino code (with comments)
• Tables comparing expected vs measured values
• Short explanations for each task

Focus is on understanding and clarity, not perfection.

# 8. Mentor Support Guidelines

Before asking for help, students should:
• Recheck wiring
• Rerun simulation
• Read error messages
• Refer to this resource document

Mentors will prioritize conceptual doubts over syntax errors.