# FINAL REPORT

BY: SAANVI SINGH

250934

Explanation:

- Downloading all required libraries. mounting google drive to in order to save the requirements.txt file, loading the dataset.

- Signal Preprocessing-

EEG data is very noisy and needs to be preprocessed to make it M.L. suitable. We have applied bandpass filter, notch filter, removed bad channels, re-referenced and done ICA followed by removing the bad components. We have also extracted the epochs.

- Feature Extraction-

We have extracted features using Xdawn and not manually.

- Classifier-

We have used classical M.L. models L.D.A., S.V.M and S.V.M. with ensemble learning as well as EEGNet(a C.N.N.) for building our speller. EEGNet code is from the 2018 paper: Lawhern et al. (2018) — EEGNet: A Compact Convolutional Neural Network for EEG-based BCIs with slight modifications to suit our data structure. We have also used class weights to handle data imbalance. Additionally, we have also used early stopping as well as reduce learning rate on plateau as without them the model was learning unstably.

- Evaluation-

We have computed classification metrics-accuracy,f-1 score,precision and recall for each model. We have also printed their confusion matrices and calculated ITR-the primary metric.

- Saving models, plots, metrics and  creating the requirements.txt file.

==================================================

==================================================

LDA Classification Report:

==================================================

precision    recall    f1-score   support

NonTarget      	   0.94          0.90       0.92         1440

Target       	   0.59          0.73        0.66         288

accuracy                                              0.87       1728

macro avg          0.77          0.81      0.79         1728

weighted avg       0.89          0.87       0.88        1728

==================================================

SVM Classification Report:

==================================================

precision    recall   f1-score   support

NonTarget       0.97      	      0.97      0.97         1440

Target       	0.87      	      0.86      0.87          288

accuracy                                           0.96         1728

macro avg       0.92             0.92       0.92        1728

weighted avg     0.96            0.96       0.96         1728

==================================================

Ensemble SVM Classification Report:

==================================================

precision    recall  f1-score   support

NonTarget       1.00      	      1.00      1.00        36

Target           1.00             1.00      1.00        36

accuracy                           	     1.00        72

macro avg       1.00     	      1.00      1.00        72

weighted avg     1.00            1.00       1.00        72

==================================================

EEGNet Classification Report:

==================================================

precision    recall     f1-score   support

NonTarget       0.83               1.00      0.91         288

Target      	 0.00     	         0.00      0.00          58

accuracy                          	        0.83       346

macro avg       0.42               0.50       0.45       346

weighted avg     0.69              0.83       0.76       346

==================================================

FINAL EVALUATION RESULTS

==================================================

Model                                 Accuracy       ITR (bits/min)

---------------------------------------------------------------------------------

LDA            		 0.872               113.21

SVM            		 0.956               133.83

EEGNet          		0.832                104.51

SVM + Ensemble  	1.000                147.71

==================================================

In terms of accuracy, L.D.A. underperforms with .87 while S.V.M, with its R.B.F. kernel and balanced class weights overshines with .96. As expected, S.V.M. with ensemble learning shows a significant improvement with a whooping 1. However, we must take this perfect score with a pinch of salt as the testing is done only on 72 characters and reliable testing requires more samples. Nonetheless, S.V.M. with ensemble learning will indeed improve accuracy by reducing both bias and variance.  EEGNet scores a low .84 and even that low .84 is misleading- target f1 is zero meaning EEGNet just predicts non target everytime. This is consistent with B.C.I. literature-classical M.L. models outperform EEGNet when the data set is small.

The primary metric, I.T.R., which rewards both accuracy and speed, is the actual yardstick for selecting the best model. S.V.M. + ensemble learning steals the show here, with 147.71 bits/min.

### CONCLUSION

SVM+ Ensemble works best for our usecase.